Damascus Transit

GPS Tracker Prototype — SIM900 (2G) + ESP32

Transport: MQTT → self-hosted Mosquitto broker

A bench-to-map prototype that publishes the required data — device ID, latitude, longitude — over MQTT, the same pipeline the fleet will use at scale.

# What you'll build

A SIM900 (2G GSM/GPRS) module gives the ESP32 mobile internet; a small GPS module gives it location. Instead of an HTTP POST, the ESP32 publishes one compact JSON message per fix to an MQTT topic on your own Mosquitto broker. A small always-on consumer subscribes to the broker and feeds every frame into the exact same ingest pipeline as the rest of the platform, so the position lands on the live map and in the database.

ℹ Why MQTT here MQTT keeps a single lightweight connection open and pushes tiny messages — ideal for many vehicles on metered 2G. The broker and the consumer run on a machine you control (a Linux VM or Raspberry Pi); they cannot run on Vercel. The phone apps keep using HTTPS — both paths feed the same live map.

# Parts

| Part | What it does | Notes |
| --- | --- | --- |
| SIMCom SIM900 (2G GSM/GPRS) | Mobile internet over 2G/GPRS | Classic 2G module. No GPS inside |
| u-blox NEO-6M GPS module | Provides latitude/longitude | ≈ $5 (NEO-M8N ≈ $10 for a faster fix) |
| ESP32 dev board | Runs the firmware, builds the JSON | Any common ESP32 |
| Nano SIM with a data plan | 2G data (Syriatel / MTN today) | A few MB/day is plenty |
| 5V / 2A power supply | Powers the SIM900 cleanly | Do NOT power the modem from the ESP32 3V3 pin |
| GPS + GSM antennas | Satellite + cellular reception | Usually included with the modules |

⚠ Power is the #1 cause of failure with 2G modules SIM900 draws short bursts up to ~2 A when transmitting. Powering it from the ESP32's 3.3 V pin will brown-out and reset the board. Use a dedicated 5 V / 2 A supply into the module's VIN (most breakouts have an onboard regulator), and share a common GND with the ESP32.

# How the data flows

NEO-6M (GPS) → ESP32 → SIM900 (2G) → Mosquitto broker → mqtt-consumer → vehicle_positions + live map

The GPS module streams the location to the ESP32. The ESP32 builds a small JSON object and publishes it to the topic vehicles/<vehicle_id>/status. Your Mosquitto broker receives it; the consumer (api.workers.mqtt_consumer) verifies, decodes, and runs the shared ingest pipeline — updating the live map (Redis) and storing the row in the database.

# Wiring

## SIM900 (2G) ↔ ESP32

| SIM900 pin | ESP32 pin | Meaning |
| --- | --- | --- |
| TXD | GPIO 26 (RX1) | module talks → ESP32 listens |
| RXD | GPIO 27 (TX1) | ESP32 talks → module listens |
| VIN / 5V | 5 V / 2 A supply | NOT the ESP32 3V3 pin |
| GND | Common GND | shared with ESP32 + PSU |

## NEO-6M GPS ↔ ESP32

| NEO-6M pin | ESP32 pin | Meaning |
| --- | --- | --- |
| TX | GPIO 16 (RX2) | GPS talks → ESP32 listens |
| RX | GPIO 17 (TX2) | ESP32 talks → GPS listens |
| VCC | 3V3 | GPS runs at 3.3 V |
| GND | GND | common ground |

# Arduino libraries

- TinyGSM — drives the 2G modem + GPRS (use the SIM800 profile)

- PubSubClient — the MQTT client that publishes over the GPRS connection

- TinyGPSPlus — parses the GPS module's NMEA output

# Stand up the broker + consumer (once)

On your always-on Linux box (VM or Raspberry Pi), from the project folder:

# 1) set .env: SUPABASE_URL, SUPABASE_SERVICE_KEY, REDIS_PUBSUB_URL, MQTT_*

# 2) start broker + consumer + redis

docker compose -f docker-compose.scale.yml up -d redis mosquitto mqtt-consumer

docker compose -f docker-compose.scale.yml logs -f mqtt-consumer

Smoke-test the whole pipeline with the simulator before touching hardware:

pip install aiomqtt

python scripts/mqtt_sim_publish.py --host <broker-host> --vehicles 5 --interval 5

✓ Full broker runbook Topic structure, payload spec, and turning on broker authentication + TLS are documented in docs/MQTT_INGEST.md in the repository.

# The firmware

Open firmware/esp32_sim900_mqtt/esp32_sim900_mqtt.ino, set VEHICLE_ID, OPERATOR_ID, APN, and MQTT_HOST (your broker's public IP/host), then flash. It publishes one JSON fix every 10 seconds. The key parts:

#define TINY_GSM_MODEM_SIM800   // SIM900 speaks the SIM800 AT set

#include <TinyGsmClient.h>

#include <PubSubClient.h>

#include <TinyGPSPlus.h>

const char* VEHICLE_ID  = "BUS-101";

const char* OPERATOR_ID = "00000000-0000-0000-0000-000000000001";

const char* APN         = "internet";            // your SIM operator's APN

const char* MQTT_HOST   = "203.0.113.10";        // your broker's public host

const uint16_t MQTT_PORT = 1883;                 // 1883 plain  /  8883 TLS

TinyGsm        modem(SerialAT);

TinyGsmClient  gsmClient(modem);

PubSubClient   mqtt(gsmClient);

TinyGPSPlus    gps;

// once per fix, every 10 s:

snprintf(payload, sizeof(payload),

"{\"vehicle_id\":\"%s\",\"operator_id\":\"%s\",\"timestamp\":%llu,"

"\"latitude\":%.6f,\"longitude\":%.6f,\"speed_kph\":%.1f,"

"\"heading\":%.1f,\"engine_state\":true}",

VEHICLE_ID, OPERATOR_ID, ts, lat, lon, kmph, course);

mqtt.setServer(MQTT_HOST, MQTT_PORT);

mqtt.connect(String("dts-") + VEHICLE_ID);       // + user/pass if broker requires

mqtt.publish("vehicles/" + String(VEHICLE_ID) + "/status", payload);

ℹ Payload = the server's _PartialDecode shape vehicle_id, timestamp (ms epoch), latitude, longitude are required. operator_id is required for the live map (Redis geo); without it the position still persists to vehicle_positions. The timestamp is computed from the GPS clock, so no RTC is needed.

# Test & verify

- Power the unit outdoors / by a window so the GPS gets a fix (the LED on the NEO-6M blinks once it locks).

- Watch the ESP32 serial monitor — you should see [pub] lines with the JSON it publishes.

- Watch the consumer logs — docker compose ... logs -f mqtt-consumer shows frames arriving.

- Open the admin live map — the paired vehicle appears and moves (operator_id + Redis required).

# Troubleshooting

| Symptom | Likely cause | Fix |
| --- | --- | --- |
| Board resets when publishing | 2G current burst browning out power | Real 5 V / 2 A supply into the module; common GND |
| [mqtt] connect... fail | Broker host/port wrong or not reachable on 2G | Use the broker's PUBLIC IP; open 1883 (or 8883) on its firewall |
| Publishes but nothing on map | Missing operator_id, or Redis/Supabase env unset on the consumer | Set OPERATOR_ID in firmware; set the consumer env |
| No GPS fix | Indoors / antenna | Move outside; wait 1–2 min for cold start |
| Can't attach / no data | Wrong APN or no SIM credit | Set the operator APN; check the SIM has a 2G data plan |

# Notes & next step

- Broker auth: the dev broker allows anonymous for local testing. Before deployment, disable anonymous and add a password (or per-device certs) — see docs/MQTT_INGEST.md — and set the same credentials in the firmware.

- TLS on 2G (port 8883) is heavy and limited on SIM900; for a reliable prototype keep plain MQTT over a trusted network/VPN, or bench-test over the ESP32's WiFi.

- 2G is being wound down globally — use SIM900 for the bench/field prototype, and a single 4G+GPS board (e.g. LilyGO T-SIM7600) for the real fleet. The broker, topic, and payload are identical — only the modem changes.

In one line: Wire a SIM900 + NEO-6M to an ESP32, power it properly, run the Mosquitto broker + consumer on your own box, flash the MQTT firmware, and it publishes deviceId + latitude + longitude every 10 seconds to the same pipeline the whole platform uses.