Damascus Transit

GPS Tracker Prototype — SIM800 (2G) + ESP32

Transport: MQTT → self-hosted Mosquitto broker

Version 2.0  ·  June 2026  ·  Transport: MQTT (self-hosted Mosquitto + mqtt-consumer)

✓ The short version SIM800 is a 2G GSM/GPRS module — it carries the data but has NO GPS, so you add a small GPS module (NEO-6M). Flash the firmware, setting the unit's fleet code (VEHICLE_ID) and operator once. It publishes a compact JSON fix every 10 s over MQTT to your Mosquitto broker — the same pipeline the fleet uses. No HTTP, no per-message signature. The operator only does: add plate → pair the fleet code → assign route. No database, no settings.

ℹ What changed in this version Transport is now MQTT instead of the HTTPS/Traccar POST. This also removes the awkward TLS-on-2G problem the old HTTP build had: the device opens one plain MQTT connection to a broker you control, rather than a TLS web request to Vercel.

# A. Parts you need

| Item | What it does | Notes |
| --- | --- | --- |
| ESP32 dev kit | The brain: reads GPS, builds + publishes data | What you already have |
| SIMCom SIM800 (2G GSM/GPRS) | Mobile internet over 2G/GPRS | What you already have. No GPS inside |
| u-blox NEO-6M GPS module | Provides latitude / longitude | ≈ $5. (NEO-M8N ≈ $10 for a faster fix) |
| Nano SIM with a data plan | 2G data (works on Syriatel / MTN today) | A few MB/day is plenty |
| GPS + GSM antennas | Satellite + cellular reception | Usually included with the modules |
| Stable 5 V / 2 A supply | Powers the SIM800 through its current bursts | Critical — see the power warning |
| Jumper wires + breadboard | Wiring it together | — |

⚠ Power is the #1 cause of failure with 2G modules SIM800 draws short bursts up to ~2 A when transmitting. Powering it from the ESP32's 3.3 V pin will brown-out and reset the board. Use a dedicated 5 V / 2 A supply into your module's VIN (most breakouts have an onboard regulator), or ~4 V direct if your board has no regulator. Share a common GND with the ESP32.

# B. How it works

The GPS module reads the location from satellites and streams it to the ESP32. The ESP32 builds a tiny JSON object and publishes it over the 2G network to your Mosquitto broker. A small always-on consumer subscribes to the broker and runs the same ingest pipeline as the rest of the platform, so the position lands on the live map and in the database.

NEO-6M → ESP32 → SIM800 (2G) → Mosquitto broker → mqtt-consumer → Live map + DB

The broker and the consumer run on one always-on box you control (a VM or a Raspberry Pi) — the public API on Vercel can't hold a long-lived MQTT subscription. Stand them up once (see section E) and every unit points at them.

# C. The data it publishes

The unit publishes to the topic vehicles/<vehicle_id>/status. Four values are required; speed, heading and engine state are optional. The vehicle_id is the fleet code you pair to the bus.

topic: vehicles/BUS-101/status (QoS 0, every 10 s) payload: {"vehicle_id":"BUS-101", "operator_id":"00000000-0000-0000-0000-000000000001", "timestamp":1718540000000, "latitude":33.510200,"longitude":36.291300, "speed_kph":0.0,"heading":0.0,"engine_state":true}

| Field | Type | Required | Meaning |
| --- | --- | --- | --- |
| vehicle_id | text | Yes | The fleet code you pair to the bus, e.g. “BUS-101”. |
| timestamp | number | Yes | Milliseconds since the Unix epoch (from the GPS UTC fix). |
| latitude | number | Yes | GPS latitude, decimal degrees. |
| longitude | number | Yes | GPS longitude, decimal degrees. |
| operator_id | text | For live map | Your operator UUID — needed for the Redis live map. |

# D. Wiring

The ESP32 has three hardware UARTs. We use one for the modem and one for the GPS (USB stays free for programming + the serial monitor). Pin numbers below are a suggestion — any free GPIOs work; match them to the firmware.

## D.1  SIM800 2G module ↔ ESP32

| Module pin | ESP32 pin | Note |
| --- | --- | --- |
| TXD | GPIO 26 (RX1) | module talks → ESP32 listens |
| RXD | GPIO 27 (TX1) | ESP32 talks → module listens |
| PWRKEY | GPIO 4 | power-on pulse (board dependent) |
| VIN / VCC | 5 V / 2 A supply | NOT the ESP32 3.3 V pin (see power warning) |
| GND | common ground | share GND with the ESP32 |

## D.2  NEO-6M GPS ↔ ESP32

| GPS pin | ESP32 pin | Note |
| --- | --- | --- |
| TX | GPIO 16 (RX2) | GPS NMEA → ESP32 |
| RX | GPIO 17 (TX2) | (optional; not required just to read) |
| VCC | 3.3 V | NEO-6M runs at 3.3 V |
| GND | common ground | share GND with the ESP32 |

# E. Stand up the broker + consumer (once)

On your always-on Linux box, from the project folder, set a .env (SUPABASE_URL, SUPABASE_SERVICE_KEY, REDIS_PUBSUB_URL, MQTT_*) then:

docker compose -f docker-compose.scale.yml up -d redis mosquitto mqtt-consumer docker compose -f docker-compose.scale.yml logs -f mqtt-consumer

Smoke-test the whole pipeline with the simulator before touching hardware:

pip install aiomqtt python scripts/mqtt_sim_publish.py --host <broker-host> --vehicles 5 --interval 5

# F. The code to flash

## F.1  Libraries (Arduino IDE → Library Manager)

- TinyGSM — drives the 2G modem + GPRS.

- PubSubClient — the MQTT client that publishes over the GPRS connection.

- TinyGPSPlus — parses the GPS module's NMEA output.

## F.2  The settings — set once per unit

const char* VEHICLE_ID = "BUS-101"; // fleet code you pair in the admin app const char* OPERATOR_ID = "00000000-0000-0000-0000-000000000001"; const char* APN = "internet"; // your SIM operator's APN const char* MQTT_HOST = "203.0.113.10";// your broker's public IP / host const uint16_t MQTT_PORT = 1883; // 1883 plain · 8883 TLS (heavy on 2G)

## F.3  Full sketch

/* Damascus Transit - GPS Tracker PROTOTYPE (TRANSPORT: MQTT) * Hardware: ESP32 + SIMCom SIM800 (2G GSM/GPRS) + u-blox NEO-6M GPS * Publishes a compact JSON fix every 10 s to the Mosquitto broker topic * vehicles/<VEHICLE_ID>/status * matching the server's _PartialDecode shape (api/routers/mqtt_ingest.py). */ // ====== SETTINGS YOU SET ONCE PER UNIT ====== const char* VEHICLE_ID = "BUS-101"; // fleet code you pair in the admin app const char* OPERATOR_ID = "00000000-0000-0000-0000-000000000001"; // operator UUID const char* APN = "internet"; // your SIM operator's APN const char* MQTT_HOST = "203.0.113.10";// your broker's PUBLIC IP / host const uint16_t MQTT_PORT = 1883; // 1883 plain · 8883 TLS (heavy on 2G) const char* MQTT_USER = ""; // set once the broker requires auth const char* MQTT_PASS = ""; const uint32_t SEND_EVERY_MS = 10000; // ============================================ #define TINY_GSM_MODEM_SIM800 // SIM900 is command-compatible: same profile #define SerialMon Serial #define SerialAT Serial1 #include <TinyGsmClient.h> #include <PubSubClient.h> // MQTT client (replaces ArduinoHttpClient) #include <TinyGPS++.h> // --- modem UART pins --- #define MODEM_RX 26 #define MODEM_TX 27 #define MODEM_PWRKEY 4 #define UART_BAUD 115200 // --- GPS UART (UART2) --- #define GPS_RX 16 #define GPS_TX 17 #define GPS_BAUD 9600 TinyGsm modem(SerialAT); TinyGsmClient gsmClient(modem); // plain TCP (TinyGsmClientSecure for 8883) PubSubClient mqtt(gsmClient); TinyGPSPlus gps; char topic[64], payload[256]; uint32_t lastSend = 0; void connectMqtt() { if (mqtt.connected()) return; String cid = String("dts-") + VEHICLE_ID; mqtt.connect(cid.c_str(), MQTT_USER, MQTT_PASS); // user/pass ignored if empty } void setup() { SerialMon.begin(115200); delay(100); Serial2.begin(GPS_BAUD, SERIAL_8N1, GPS_RX, GPS_TX); // NEO-6M on UART2 pinMode(MODEM_PWRKEY, OUTPUT); digitalWrite(MODEM_PWRKEY, LOW); delay(1000); digitalWrite(MODEM_PWRKEY, HIGH); SerialAT.begin(UART_BAUD, SERIAL_8N1, MODEM_RX, MODEM_TX); delay(3000); modem.restart(); modem.waitForNetwork(60000L); modem.gprsConnect(APN, "", ""); SerialMon.println(modem.isGprsConnected() ? "Online" : "No data"); mqtt.setServer(MQTT_HOST, MQTT_PORT); snprintf(topic, sizeof(topic), "vehicles/%s/status", VEHICLE_ID); } void loop() { while (Serial2.available()) gps.encode(Serial2.read()); // feed the parser if (millis() - lastSend < SEND_EVERY_MS) return; lastSend = millis(); if (!gps.location.isValid()) { SerialMon.println("Waiting for GPS fix..."); return; } if (!modem.isGprsConnected()) modem.gprsConnect(APN, "", ""); connectMqtt(); mqtt.loop(); // timestamp from the GPS UTC fix -> ms since epoch unsigned long long ts = 0; if (gps.date.isValid() && gps.time.isValid()) { struct tm t = {0}; t.tm_year = gps.date.year() - 1900; t.tm_mon = gps.date.month() - 1; t.tm_mday = gps.date.day(); t.tm_hour = gps.time.hour(); t.tm_min = gps.time.minute(); t.tm_sec = gps.time.second(); ts = (unsigned long long)mktime(&t) * 1000ULL; // treat fields as UTC } snprintf(payload, sizeof(payload), "{\"vehicle_id\":\"%s\",\"operator_id\":\"%s\",\"timestamp\":%llu," "\"latitude\":%.6f,\"longitude\":%.6f,\"speed_kph\":%.1f," "\"heading\":%.1f,\"engine_state\":true}", VEHICLE_ID, OPERATOR_ID, ts, gps.location.lat(), gps.location.lng(), gps.speed.isValid() ? gps.speed.kmph() : 0.0, gps.course.isValid() ? gps.course.deg() : 0.0); bool ok = mqtt.publish(topic, payload); SerialMon.print(ok ? "[pub] " : "[pub failed] "); SerialMon.println(payload); }

## F.4  Flashing

- Install the Arduino IDE and add ESP32 board support (Boards Manager → ‘esp32’).

- Install the three libraries above.

- Open the sketch, set VEHICLE_ID / OPERATOR_ID / APN / MQTT_HOST, select your ESP32 board, plug in via USB, and click Upload.

- Open Serial Monitor (115200). You should see ‘Online’, then [pub] lines with the JSON it publishes.

# G. The easy part for operators

Operators never touch the broker, the database, or the hardware. In the admin app:

- Open Vehicles → ‘Register new vehicle’.

- Type the plate / fleet code — use the SAME code as the unit's VEHICLE_ID — plus name, type and capacity.

- Save, then assign the bus to a route. An admin approves it if approval is on.

From then on, the unit's messages (which carry that vehicle_id) are matched to that bus automatically and it appears on the live map.

# H. Troubleshooting

| Symptom | Likely cause | Fix |
| --- | --- | --- |
| Board resets when publishing | 2G current burst browning out power | Use a real 5 V / 2 A supply into the module; common GND. |
| ‘Waiting for GPS fix’ forever | GPS antenna can't see the sky | Put the GPS antenna by a window / outdoors; first fix can take 1–2 min. |
| Serial shows [pub failed] | Not connected to the broker | Check MQTT_HOST is the broker's public IP, the port is open, and any username/password match. |
| Bus never appears on the map | Not paired, or operator_id missing | Pair the unit's VEHICLE_ID to a bus; make sure OPERATOR_ID is set in the firmware. |
| Frames in logs but no map dot | Bus pending / suspended | Admin approves the bus; it must be active. |
| No data / can't attach | Wrong APN or no SIM credit | Set the operator APN; check the SIM has a data plan + 2G enabled. |

⚠ TLS on 2G — keep the prototype plain Port 8883 (TLS) is heavy and patchy on SIM800-class modules. For a reliable bench/field prototype, keep plain MQTT (1883) on a trusted network or VPN, or bench-test over the ESP32's built-in WiFi (WiFiClient) which does modern TLS effortlessly. Treat the 2G path as the cellular field test, and secure the broker with a username/password.

# I. Moving to production

- 2G works in Syria today, but networks are prioritising 3G/4G and 2G is being wound down globally — don't build the whole fleet on 2G.

- For real on-bus units, use a single LilyGO T-SIM7600 board (ESP32 + 4G LTE + GPS in one): solid TLS, built-in GPS, less wiring. See the main GPS Hardware & Setup Guide.

- The broker, the topic, the JSON payload, and the operator pairing flow are all identical — only the on-bus modem changes.

✓ Summary Wire SIM800 + a NEO-6M GPS to your ESP32, power it properly, run the Mosquitto broker + consumer on your own box, flash the MQTT sketch, and pair from the admin app. It publishes vehicle_id + latitude + longitude every 10 seconds to vehicles/<id>/status — the same pipeline the whole platform uses. Move to a 4G+GPS board for the real fleet.