Damascus Transit

GPS Tracker — Hardware & Setup Guide

Transport: MQTT → self-hosted Mosquitto broker

Version 2.0  ·  June 2026  ·  Transport: MQTT (self-hosted Mosquitto + mqtt-consumer)

✓ The short version You build one small box per bus (ESP32 + 4G + GPS — a single ready-made board). You flash the SAME firmware to every box; you set its fleet code (VEHICLE_ID) and operator once before flashing. The box publishes its GPS over MQTT to your own Mosquitto broker every 10 seconds — the same pipeline the whole fleet uses. The operator does only 3 things per bus: type the plate, pair the box's fleet code, pick the route. ~1 minute. No database, no wiring.

ℹ What changed in this version Transport is now MQTT, not the HTTP/Traccar webhook. The device publishes JSON to vehicles/<id>/status on a broker you run; a small always-on consumer feeds it into the live map and database. No per-message HMAC signature — the broker authenticates the device instead. The phone apps (passenger/driver) keep using the HTTPS API; MQTT is only the path for the on-bus hardware. Both end up on the same live map.

# 1. How it all fits together

Each bus carries a small tracker. It reads its location from GPS satellites and publishes it over the mobile network (4G) to your MQTT broker every few seconds. A small always-on consumer subscribes to the broker, checks each message, finds which bus it belongs to, and updates the live map that passengers, operators and admins see.

GPS satellites → Tracker → 4G → Mosquitto broker → mqtt-consumer → Live map + DB

Why a broker and a consumer, not a direct web call? The public API runs on Vercel (serverless), where a long-lived MQTT subscription cannot live. So the broker AND the consumer run on an always-on host you control — a small Linux VM, a Raspberry Pi, or the ministry's own server. The consumer reuses the exact same ingest code as before (api/routers/mqtt_ingest.py), so only the transport changes; the live map and database behave identically.

The tracker identifies itself with the fleet code you set in its firmware (VEHICLE_ID, e.g. “B-104”) and your operator UUID. The operator simply pairs that same code to the bus in the admin app — so there is still nothing to type into a database.

# 2. Part A — The hardware to build

The simplest, most reliable option is a single board that already contains the ESP32, the 4G modem, and the GPS receiver together. You do not need to wire separate modules. The recommended board is the LilyGO T-SIM7600 (ESP32 + 4G LTE + GPS). The transport (MQTT) does not change the hardware at all.

## 2.1 Bill of materials (per tracker)

| Item | What it does | Notes / est. cost (USD) |
| --- | --- | --- |
| LilyGO T-SIM7600G-H (ESP32 + 4G LTE + GPS) | The whole brain: reads GPS and sends data over 4G | All-in-one board. ~ $45–60 |
| LTE antenna + GPS antenna | Cellular signal + satellite reception | Usually included with the board. ~ $5 |
| Nano SIM card with a data plan | Mobile internet for the box | Syriatel or MTN Syria. A few hundred MB/month is plenty |
| Power: 12 V → 5 V USB adapter (car type) | Runs the box from the bus battery | Any quality 12–24 V to USB-C car adapter. ~ $5 |
| Optional: small LiPo battery (3.7 V) | Keeps reporting if engine power drops | Optional buffer. ~ $5 |
| Small enclosure + mount | Protects the box on the dashboard | Any small project box. ~ $3 |

Rough total: about $60–80 per bus in parts, plus a small monthly SIM data fee.

## 2.2 Assembly — there is almost nothing to wire

- Insert the SIM card into the board's SIM slot.

- Screw on the two antennas (one for LTE/cellular, one for GPS). Place the GPS antenna where it can ‘see’ the sky (top of the dashboard).

- Power the board from the bus: plug the 12 V→USB adapter into the board's USB-C port. (Optionally clip in the small LiPo as a buffer.)

- Put it in the enclosure and mount it on the dashboard.

# 3. Part B — The data: what the device publishes

The tracker publishes one small JSON message to an MQTT topic every few seconds. There is no HTTP request and no Traccar server in the middle — the device publishes straight to your Mosquitto broker, and the consumer does the rest.

## 3.1 Where it publishes

| Field | Value |
| --- | --- |
| Broker | Your Mosquitto broker (docker-compose.scale.yml), reachable at its public IP/host |
| Port | 1883 (plain) or 8883 (TLS, recommended for production) |
| Status topic | vehicles/<vehicle_id>/status (QoS 0, every 10 s) |
| Event topic | vehicles/<vehicle_id>/event (QoS 1, e.g. ignition-off) |
| Auth | Broker username/password (or per-device X.509 cert). No per-message signature. |

## 3.2 The JSON body (the exact format)

Publish a small JSON object matching the server's _PartialDecode shape. Four values are required; the rest are optional.

{ "vehicle_id": "BUS-101", "operator_id": "00000000-0000-0000-0000-000000000001", "timestamp": 1718540000000, "latitude": 33.510200, "longitude": 36.291300, "speed_kph": 42.0, "heading": 180.0, "engine_state": true }

| Key | Type | Required | Meaning |
| --- | --- | --- | --- |
| vehicle_id | text | Yes | The fleet code you pair to the bus, e.g. “BUS-101”. Also the <id> in the topic. |
| timestamp | number | Yes | Milliseconds since the Unix epoch, from the GPS UTC fix. |
| latitude | number | Yes | GPS latitude, decimal degrees (e.g. 33.5102). |
| longitude | number | Yes | GPS longitude, decimal degrees (e.g. 36.2913). |
| operator_id | text | For live map | Your operator UUID. Without it the position still persists, but won't show on the Redis live map. |
| speed_kph | number | Optional | Speed in km/h (default 0). |
| heading | number | Optional | Direction of travel 0–360° (default 0). |
| engine_state | bool | Optional | true while the ignition is on. |

A JSON array, or a {"frames":[…]} batch, is also accepted — handy when the device drained an offline buffer.

## 3.3 How often to publish

Every 5–10 seconds while the bus is moving is a good balance of a smooth live map and low data use. At 10 seconds, each tracker uses only a few megabytes per day.

# 4. Part C — The code to flash into the ESP32

This is the complete MQTT firmware. You set the fleet code (VEHICLE_ID), operator UUID, APN and broker host once per unit at the top, then flash.

## 4.1 What the firmware does

- Starts the 4G modem and connects to mobile internet.

- Turns on the GPS and waits for a fix.

- Connects to your Mosquitto broker (username/password if you set them).

- Every 10 seconds: builds the JSON and publishes it to vehicles/<VEHICLE_ID>/status.

## 4.2 The settings — set once per unit

const char* VEHICLE_ID = "BUS-101"; // fleet code you pair in the admin app const char* OPERATOR_ID = "00000000-0000-0000-0000-000000000001"; const char* APN = "internet"; // your SIM operator's APN const char* MQTT_HOST = "203.0.113.10";// your broker's public IP / host const uint16_t MQTT_PORT = 1883; // 1883 plain · 8883 TLS const char* MQTT_USER = ""; // set once the broker requires auth const char* MQTT_PASS = "";

## 4.3 Libraries to install (Arduino IDE → Library Manager)

- TinyGSM — talks to the 4G modem and the GPS.

- PubSubClient — the MQTT client (replaces ArduinoHttpClient).

- No HMAC library is needed any more — security is handled by the broker, not a per-message signature.

## 4.4 The full sketch

/* Damascus Transit - GPS Tracker firmware (TRANSPORT: MQTT) * Board: LilyGO T-SIM7600 (ESP32 + 4G LTE + GPS) * Publishes one compact JSON fix every 10 s to the Mosquitto broker topic * vehicles/<VEHICLE_ID>/status * matching the server's _PartialDecode shape (api/routers/mqtt_ingest.py), * so MQTT reuses the EXACT same ingest pipeline as the old HTTP bridge. */ // ====== SETTINGS YOU SET ONCE PER UNIT ====== const char* VEHICLE_ID = "BUS-101"; // the fleet code you pair in the admin app const char* OPERATOR_ID = "00000000-0000-0000-0000-000000000001"; // your operator UUID const char* APN = "internet"; // your SIM operator's APN const char* MQTT_HOST = "203.0.113.10";// your broker's PUBLIC IP / hostname const uint16_t MQTT_PORT = 1883; // 1883 plain · 8883 TLS const char* MQTT_USER = ""; // set once the broker requires auth const char* MQTT_PASS = ""; const uint32_t SEND_EVERY_MS = 10000; // 10 seconds // ============================================ #define TINY_GSM_MODEM_SIM7600 #define SerialMon Serial #define SerialAT Serial1 #include <TinyGsmClient.h> #include <PubSubClient.h> // MQTT client (replaces ArduinoHttpClient) // --- LilyGO T-SIM7600 pins (adjust if your board revision differs) --- #define MODEM_TX 27 #define MODEM_RX 26 #define MODEM_PWRKEY 4 #define UART_BAUD 115200 TinyGsm modem(SerialAT); TinyGsmClient gsmClient(modem); // plain TCP. For 8883 use TinyGsmClientSecure. PubSubClient mqtt(gsmClient); char topic[64]; char payload[256]; void connectMqtt() { if (mqtt.connected()) return; String cid = String("dts-") + VEHICLE_ID; // unique client id mqtt.connect(cid.c_str(), MQTT_USER, MQTT_PASS); // user/pass ignored if empty } void setup() { SerialMon.begin(115200); delay(100); pinMode(MODEM_PWRKEY, OUTPUT); digitalWrite(MODEM_PWRKEY, LOW); delay(1000); digitalWrite(MODEM_PWRKEY, HIGH); SerialAT.begin(UART_BAUD, SERIAL_8N1, MODEM_RX, MODEM_TX); delay(3000); modem.restart(); modem.waitForNetwork(60000L); modem.gprsConnect(APN, "", ""); SerialMon.println(modem.isGprsConnected() ? "Online" : "No data"); modem.enableGPS(); // turn the GPS receiver on mqtt.setServer(MQTT_HOST, MQTT_PORT); snprintf(topic, sizeof(topic), "vehicles/%s/status", VEHICLE_ID); } void loop() { if (!modem.isGprsConnected()) modem.gprsConnect(APN, "", ""); connectMqtt(); mqtt.loop(); float lat, lon, spd = 0, course = 0, alt = 0, acc = 0; int vsat = 0, usat = 0; // T-SIM7600 returns the GPS UTC time too; use it for the timestamp (ms). int yr, mo, dy, hr, mi, se; if (modem.getGPS(&lat, &lon, &spd, &alt, &vsat, &usat, &acc, &yr, &mo, &dy, &hr, &mi, &se)) { struct tm t = {0}; t.tm_year = yr - 1900; t.tm_mon = mo - 1; t.tm_mday = dy; t.tm_hour = hr; t.tm_min = mi; t.tm_sec = se; unsigned long long ts = (unsigned long long)mktime(&t) * 1000ULL; // UTC->ms snprintf(payload, sizeof(payload), "{\"vehicle_id\":\"%s\",\"operator_id\":\"%s\",\"timestamp\":%llu," "\"latitude\":%.6f,\"longitude\":%.6f,\"speed_kph\":%.1f," "\"heading\":%.1f,\"engine_state\":true}", VEHICLE_ID, OPERATOR_ID, ts, lat, lon, spd, course); bool ok = mqtt.publish(topic, payload); SerialMon.println(ok ? String("[pub] ") + payload : "[pub failed]"); } else { SerialMon.println("Waiting for GPS fix..."); } delay(SEND_EVERY_MS); }

## 4.5 Flashing it

- Install the Arduino IDE and add ESP32 board support (Boards Manager → ‘esp32’).

- Install the two libraries above.

- Open the sketch, set VEHICLE_ID / OPERATOR_ID / APN / MQTT_HOST, select the ESP32 board, plug in via USB, and click Upload.

- Open Serial Monitor (115200). You should see ‘Online’, then [pub] lines with the JSON it publishes.

# 5. Part D — Stand up the broker + consumer (once)

The broker and the consumer run on one always-on Linux box you control (a small VM, a Raspberry Pi, or the ministry server). You do this one time; every tracker then points at it.

- On the box, create a .env next to docker-compose.scale.yml with: SUPABASE_URL, SUPABASE_SERVICE_KEY, REDIS_PUBSUB_URL, MQTT_BROKER_HOST, MQTT_BROKER_PORT, MQTT_STATUS_TOPIC=vehicles/+/status.

- Start the broker, consumer and Redis, then watch the logs:

docker compose -f docker-compose.scale.yml up -d redis mosquitto mqtt-consumer docker compose -f docker-compose.scale.yml logs -f mqtt-consumer

Smoke-test the whole pipeline with the simulator before touching hardware — you should see frames flow and test vehicles appear on the admin map:

pip install aiomqtt python scripts/mqtt_sim_publish.py --host <broker-host> --vehicles 5 --interval 5

⚠ Secure the broker before any real deployment The dev broker allows anonymous connections for local testing. Before production: set allow_anonymous false, add a password file (mosquitto_passwd), set MQTT_USERNAME / MQTT_PASSWORD for the consumer and the same in the firmware, and prefer TLS on 8883. On a trusted network/VPN, plain MQTT is an acceptable prototype path. See docs/MQTT_INGEST.md.

# 6. Part E — The easy part for operators

Operators never see the broker, the database, or the firmware. They work entirely inside the admin screen they already use.

## 6.1 What the operator does (about one minute)

- Open the admin app → Vehicles → ‘Register new vehicle’.

- Type the plate / fleet code (e.g. B-104) — use the SAME code as the unit's VEHICLE_ID — plus name, type and capacity.

- Save. Assign the bus to a route (and a driver if used).

- An admin approves the bus if approval is on. Done.

From that moment, the tracker's messages (which carry that same vehicle_id) are matched to this bus automatically and it appears on the live map.

# 7. Part F — From box to live map: full checklist

| Step | Who | What |
| --- | --- | --- |
| 1. Broker | You, once | Stand up Mosquitto + mqtt-consumer + Redis on your always-on box (Part D). Smoke-test with the simulator. |
| 2. Prepare a unit | You / workshop | Set VEHICLE_ID, OPERATOR_ID, APN and MQTT_HOST; flash the firmware; insert SIM; attach antennas; mount; power from the bus. |
| 3. Onboard the bus | Operator, ~1 min | Register the vehicle using the same fleet code; assign the route; admin approves if approval is on. |
| 4. Verify | You | Within ~30 s the unit gets a GPS fix; [pub] lines appear on serial, frames appear in the consumer logs, and the bus moves on the live map. |

# 8. Part G — Troubleshooting

| Symptom | Likely cause | Fix |
| --- | --- | --- |
| Bus never appears on the map | Vehicle not paired, or operator_id missing | Pair the unit's VEHICLE_ID to a bus; make sure the firmware OPERATOR_ID is set (Redis live map needs it). |
| Frames in logs but no map dot | Bus pending / not approved | An admin approves the bus, and it must be active. |
| Serial shows [pub failed] | Not connected to the broker | Check MQTT_HOST is the broker's PUBLIC IP, the port is open, and any username/password match the broker. |
| ‘Waiting for GPS fix’ forever | GPS antenna can't see sky | Move the GPS antenna to the dashboard / window; allow 1–2 min on first start. |
| ‘No data’ / can't connect | Wrong APN or no SIM credit | Set the correct operator APN; check the SIM has a data plan. |
| Connection drops on 8883 | TLS misconfigured | Verify the broker's certificate and that the firmware uses TinyGsmClientSecure; fall back to 1883 on a trusted network to isolate. |
| Map jumpy or laggy | Publish interval too long/short | 10 seconds is the sweet spot; lower only if the data budget allows. |

# 9. Cost & scaling (rough)

- Hardware: ~ $60–80 per bus, one time.

- SIM data: a small monthly fee per bus (only a few MB/day at 10-second updates).

- Broker box: one small always-on VM / Pi for the whole fleet — the only new piece MQTT adds.

- Scaling: every box runs the same firmware and self-identifies by its fleet code, so going from 1 to 100 buses is just ‘flash, label, pair’ repeated. The broker, topic, and payload stay identical whether the modem is 4G or 2G.

✓ Summary Buy the all-in-one 4G board, flash the MQTT firmware with each unit's fleet code, run one Mosquitto broker + consumer on your own box, and let operators pair a bus by its code. The device publishes a tiny JSON message every 10 seconds to vehicles/<id>/status — the same pipeline the whole platform uses.